GENERAL INFORMATION

Tables & Figures

MAIN PARTICULARS AND STABILITY DATA OF THE VESSEL

| DESIGNATION | SYMBOL | MAGNITUDE |  |  | UNIT |
| --- | --- | --- | --- | --- | --- |
|  |  | V-shape aft body desired/realised | Pram aft body |  |  |
|  |  |  | First draft | Second draft |  |
| Main particulars |  |  |  |  |  |
| Length between perpendiculars | LPP | 262.0 | 262.0 |  | M |
| Breadth | B | 40.0 | 40.0 |  | M |
| Depth | D | 24.45 | 24.45 |  | M |
| Draught moulded on FP | TF | 11.719 | 11.719 | 11.500 | M |
| Draught moulded on AP | TA | 12.856 | 12.856 | 11.500 | M |
| Displacement weight | ∆ | 76,02 | 69,441 |  | tonnes |
| Centre of gravity forward of station 0 | LCG | 123.13 | 125.52 |  | M |
| Centre of gravity above keel | KG | 17.54 | 17.54 |  | M |
| Transverse metacentric height | gmt | 1.973/1.90 | 1.79 | 1.965 | M |
| Transverse radius of gyration in air | kXX | 16.73 | 16.73 |  | M |
| Longitudinal radius of gyration in air | KYY | 62.55 | 62.55 |  | M |
| Natural roll period | T | 25.7/25.14 | 27.4 | 25.14 | S |

MAIN PARTICULARS of propellers

| DESIGNATION | SYMBOL | MAGNITUDE | UNIT |
| --- | --- | --- | --- |
| Diameter | D | 8.40 | M |
| Pitch ratio at 0.7R | P0.7/D | 0.743 | - |
| Boss- diameter ratio | d/D | 0.164 | - |
| Expanded blade area ratio | AE/A0 | 0.590 |  |
| Direction of rotation |  |  |  |

MAIN PARTICULARS of BILGE KEELS

| DESIGNATION | SYMBOL | MAGNITUDE | UNIT |
| --- | --- | --- | --- |
| Bilge keels (see also figure) |  |  |  |
| Total length | LBK | 76.54 | M |
| Height | HBK | 0.40 | M |
| Description: split in parts |  |  |  |
| Location: from 88.47 m to 165.0 m with respect to APP |  |  |  |

DESIGNATION, NOTATION, SIGN CONVENTION AND MEASURING DEVICES OF MEASURED QUANTITIES

Sample frequency 100 Hz.

| Designation | Notation |  | Positive for: | Measured by: |
| --- | --- | --- | --- | --- |
| Wave elevation: |  |  |  |  |
| Incident wave height | WAVE 1 WAVE 2 |  | Wave elevation upwards | Resistance type wave probe |
| Motions of the ship at position measuring target: |  |  |  |  |
| Surge Sway Heave Roll Pitch Yaw | X-REL Y-REL Z-REL ROLL PITCH YAW |  | Ship forwards Ship to port side Ship upwards Starboard down Bow down Bow to port side | Optical tracking system |
| Relative motions (see also figure ???): |  |  |  |  |
| At station 0.5 At station 1.5 | REL ST 0,5 REL ST 1,5 |  | Ship immersed | Resistance type wave probe |
| Rudder angle: |  |  |  |  |
| Rudder angle | RUD ANG |  | Rudder nose to port side | Servo |
| Accelerations at top (222.30 m forward of station 0, on centre line and 40.7 m above keel): |  |  |  |  |
| Longitudinal acceleration Transverse acceleration Vertical acceleration |  | AX TOP AY TOP AZ TOP | Acceleration forwards Acceleration to port side Acceleration upwards | Accelerometer |
| Accelerations at stern (43.28 m forward of station 0, on centre line and 7.15 m above keel for V-shape aft body, 32.0 m forward of station 0, on centre line and 12.65 m above keel for pram aft body ): |  |  |  |  |
| Transverse acceleration Vertical acceleration |  | AY STERN AZ STERN | Acceleration to port side Acceleration upwards | Accelerometer |
| Accelerations at bottom (222.30 m forward of station 0, on centre line and 5.50 m above keel): |  |  |  |  |
| Transverse acceleration |  | AY BOTTOM | Acceleration to port side | Accelerometer |

DESIGNATION, NOTATION AND SIGN CONVENTION OF CALCULATED QUANTITIES

Sample frequency 100 Hz.

| Designation | Notation | Positive for: |
| --- | --- | --- |
| Motions at COG (earth-fixed): |  |  |
| Low frequent surge Wave frequent surge Low frerquent sway Wave frequent sway Low frequent yaw Wave frequent yaw | SURGE LF SURGE WF SWAY LF SWAY WF YAW LF YAW WF | Ship forwards Ship forwards Ship to port side Ship to port side Bow to port side Bow to port side |

### REVIEW OF ROLL DECAY TESTS

| MARIN test no. | Speed [knots] | Remarks |
| --- | --- | --- |
| 303001 | 0 | V-shape aft body |
| 303002 | 5 | V-shape aft body |
| 303003 | 0 | Pram aft body, first draft (T=12.3m) |
| 303004 | 5 | Pram aft body, first draft (T=12.3m) |
| 310001 | 0 | Pram aft body, second draft (T=11.5m) |
| 310002 | 5 | Pram aft body, second draft (T=11.5m) |

### TESTS IN IRREGULAR HEAD SEAS – V-SHAPE AFT BODY

| MARIN test no. | Wave conditions |  |  | Speed [knots] | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | Significant wave height [m] | Peak period [s] | Heading [deg] |  |  |
| 306001 | 7.5 | 11.0 | 180 | 5 |  |
| 304001 | 7.5 | 12.6 | 180 | 5 |  |
| 305005 | 4.125 | 14.4 | 180 | 5 |  |
| 305004 | 5.25 | 14.4 | 180 | 5 |  |
| 305002 | 6.38 | 14.4 | 180 | 5 |  |
| 305003 | 6.38 | 14.4 | 180 | 5 | Same as 305002, but lower RPM |
| 305001/ 305006 | 7.5 | 14.4 | 180 | 5 |  |

### TESTS IN IRREGULAR HEAD SEAS – PRAM AFT BODY – FIRST DRAFT (T=12.3 m)

| MARIN test no. | Wave conditions |  |  | Speed [knots] | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | Significant wave height [m] | Peak period [s] | Heading [deg] |  |  |
| 308001 | 7.5 | 12.6 | 180 | 5 |  |
| 307002 | 3.0 | 14.4 | 180 | 5 |  |
| 307003 | 3.5 | 14.4 | 180 | 5 |  |
| 307001 | 4.125 | 14.4 | 180 | 5 |  |
| 307004 | 5.25 | 14.4 | 180 | 5 |  |
| 307005 | 7.5 | 14.4 | 180 | 5 |  |
| 307006/307007 | 7.5 | 14.4 | 180 | 10 |  |
| 309001 | 7.5 | 15.5 | 180 | 5 |  |

### TESTS IN IRREGULAR HEAD SEAS – PRAM AFT BODY – SECOND DRAFT (T=11.5 m)

| MARIN test no. | Wave conditions |  |  | Speed [knots] | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | Significant wave height [m] | Peak period [s] | Heading [deg] |  |  |
| 312001 | 7.5 | 12.6 | 180 | 5 |  |
| 313001 | 3.0 | 14.4 | 180 | 5 |  |
| 314001 | 3.5 | 14.4 | 180 | 5 |  |
| 315001 | 4.125 | 14.4 | 180 | 5 |  |
| 316001 | 5.25 | 14.4 | 180 | 5 |  |
| 317001 | 7.5 | 14.4 | 180 | 5 |  |
| 318001 | 7.5 | 14.4 | 180 | 10 |  |
| 311001 | 7.5 | 15.5 | 180 | 5 |  |

GENERAL ARRANGEMENT AND SMALL SCALE BODY PLAN MODEL 8004-2

LOCATION OF MEASURING DEVICES

RUDDER AND PROPELLER ARRANGEMENT

PARTICULARS AND LOCATION OF THE BILGE KEELS

NEW SEAKEEPING & MANOEUVRING BASIN

Dimensions

170  40  5 m. The basin is mainly designed for performing seakeeping, manoeuvring and still water tests on models of seagoing ships and structures.

Carriage

The carriage with a maximum speed of 6 m/s will run over the total length of the tank and consists of:

- A mainframe, spanning the full width of the basin;

- A sub frame, which will have a maximum speed of 4 m/s along this mainframe. The carriage can follow all movements of the model in the horizontal plane. With an extra installed turntable, the system will have rotating arm capability.

Waves

The basin will be fitted out with the most advanced equipment for generating wave types required in modern model testing. At two adjacent sides of the basin, segmented wave generators consisting of hinged flaps will be installed. Each flap is controlled separately by its own driving motor and has a width of 60 cm. The capacity of the wave generator will be up to a significant wave height of 0.45 m at a peak period of 2 seconds. Opposite the wave generator, passive sinkable wave absorbers will be installed. The wave generator system will be equipped with an active wave reflection compensation feature and higher order wave synthesis techniques.

Wind

Wind can be simulated by an adjustable 10 m wide platform with electrical fans.

Motion Control and Dynamic Tracking

Free running tests are performed such that the model follows an arbitrary pre-defined track through the basin (straight or curved). The carriage follows the model during this task. Deviations from the pre-defined track are minimised by means of a dynamic positioning feedback loop which controls the propulsion units, additional thrusters and steering within a particular control scenario.

Motion control is realised by means of a feedback loop with activates related stabilisation systems (fins, foils, rudders, et cetera).

Model size range

- Model length of 2 - 8 m for moving objects and up to 10 m for moored objects.

- Floating structures of any kind, size depending on water depth and wave conditions.

Tests to be performed

- Seakeeping tests of ships in waves and wind from arbitrary direction.

- Resistance and self propulsion tests of ship in calm water and in waves.

- Oscillation (PMM) and rotating arm tests in calm water and waves with a restrained model to determine hydrodynamic coefficients.

- Captive or free sailing manoeuvring tests in calm water and waves.

- Installation and sea transport tests (of offshore constructions).

- Tests on moored or fixed objects to determine the motions and loads due to waves and wind.