Table 2  Control parameters of capsizing model runs of Ship A-2

| # nominal Froude number | # Fn | 0.3 | 0.43 | 0.3 | 0.43 |
| --- | --- | --- | --- | --- | --- |
| auto pilot course from the wave direction | c(degree) | -30.0 | -10.0 | -30.0 | -30.0 |
| wave steepness | / | 1/10 | 1/10 | 1/8.7 | 1/8.7 |
| wave length to ship length ratio | /Lpp | 1.637 | 1.637 | 1.127 | 1.127 |
| proportional gain | KP | 1.0 | 1.0 | 1.0 | 1.0 |
| differential gain in full scale | ## KR(sec) | 0.0 | 0.0 | 0.0 | 0.0 |
| differential gain in model scale | KR(sec) | 0.0 | 0.0 | 0.0 | 0.0 |
| data file name for time history |  | EXP87 | EXP101 | EXP107 | EXP145 |

Here the propeller revolution is indicated by the nominal Froude number, which is the Froude number when the ship runs in calm water with the specified propeller revolution. The algorithm of the auto pilot is as follows: d=-KPc−cc−KRr

where ,  and r are the rudder angle, heading angle and yaw rate, respectively.

Table  3   Initial Conditions of free-running model runs of Ship A-2 in model scale

| # nominal Froude number | # Fn | 0.3 | 0.43 | 0.3 | 0.43 |
| --- | --- | --- | --- | --- | --- |
| auto pilot course | c(degree) | -30.0 | -10.0 | -30.0 | -30.0 |
| nondimensional longitudinal position from a wave trough | G/ | 0.863 | 0.033 | 0.875 | 0.130 |
| ship velocity in x direction | u(m/sec) | 0.254 | 0.254 | 0.214 | 0.214 |
| ship velocity in y direction | v(m/sec) | 0.0 | 0.0 | 0.0 | 0.0 |
| nondimensional vertical position from the still water level | G/a | 0.301 | 0.502 | 0.160 | 0.099 |
| ship velocity in z direction | w(m/sec) | -0.236 | -0.012 | -0.094 | 0.089 |
| roll angle | (degree) | 6.22 | -5.49 | -0.115 | -1.41 |
| roll angular velocity | p(degree/sec) | 1.15 | 11.56 | 21.36 | -6.95 |
| pitch angle | (degree) | -8.17 | -0.52 | -5.99 | 1.95 |
| pitch angular velocity | q(degree/sec) | -15.0 | -38.0 | -20.6 | -21.0 |
| yaw angle from the wave direction | (degree) | 7.08 | -29.8 | -21.6 | -19.1 |
| yaw angular velocity | r(degree/sec) | -5.15 | 4.61 | -18.31 | 9.02 |
| rudder angle | (degree) | 10.5 | 12.1 | -10.1 | -6.0 |

Here the value of v is assumed to be zero, because this was not measured in the experiment. The values of G, u, G, and w are estimated from the measured value of  and q with the assumption of low encounter frequency.  and a are wave length and amplitude, respectively. The definition of the above valued are based on the attached co-ordinate systems.